AEROSYS AVIONICS SYSTEMS LTD.

AeroSys 9000 Integrated Avionics Platform

SOFTWARE VERIFICATION CASES AND PROCEDURES

(SVCP)

| Document Number | AEROSYS-SVCP-001 |
| --- | --- |
| Revision | A — Initial Baseline |
| Date | 2026-03-14 |
| DO-178C Level | DAL B/C Tests |
| Classification | PROPRIETARY — RESTRICTED |

# 1  Purpose

This document defines the verification test cases and execution procedures for the AeroSys 9000 LLTs and HLTs. Each test case traces to one or more HLRs from AEROSYS-SRS-001. Test results are recorded in AEROSYS-SVTR-001.

All test cases are implemented in aerosys-tests.adb and executed using the AUnit framework with GNATcoverage instrumentation.

# 2  Test Environment Setup

Before executing any test cases, verify the following environment configuration:

- GNAT Pro 24.0 is installed and accessible as gnat on the PATH

- GNATcoverage 24.0 is installed: gnatcov --version shows 24.0

- Build the test binary: gprbuild -P aerosys_tests.gpr -Xmode=coverage -j0

- Verify binary was created: ls -la bin/aerosys_tests

- Record the SHA-256 of the test binary: sha256sum bin/aerosys_tests

# 3  ARINC 429 Test Cases

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-BUS-001 | HLR-BUS-001 | PASS | Word Encode/Decode Round-Trip |
| Prerequisites: AeroSys.SPARK.ARINC compiled with -Xmode=coverage Procedure: Call Encode_Word(label=0x31, SDI=1, data=0x12345, SSM=3). Decode the result with Decode_Word. Assert D.Valid=True, D.Data=0x12345, D.SSM=3, D.SDI=1. Expected: All assertions pass. Decoded fields match encoded inputs exactly. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-BUS-002 | HLR-BUS-002 | PASS | Single-Bit Error Parity Detection |
| Prerequisites: TC-BUS-001 passed Procedure: Encode a valid word. XOR bit 1 to corrupt it. Assert Check_Parity(corrupt_word)=False. Assert Check_Parity(original_word)=True. Expected: Parity check correctly distinguishes valid from corrupt word. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-BUS-003 | HLR-BUS-003 | PASS | Label Bit Reversal Involution |
| Prerequisites: None Procedure: For labels 0x00, 0x31, 0xFF, 0xAA, 0x55: Assert Reverse_Label(Reverse_Label(L))=L. Assert Reverse_Label(0x31)=0x8C. Expected: Involution property holds for all test labels. 0x31 reverses to 0x8C. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-BUS-004 | HLR-BUS-004,005 | PASS | BNR Boundary Values |
| Prerequisites: None Procedure: Test: (1) Value=0.0 → data=0. (2) Value=84.6%, res=0.00391 → decoded within ±0.01 of 84.6. (3) Encode_BNR(-2.5, 0.00137) → Decode_BNR result < 0. (4) SSM_FW → 0.0. (5) SSM_NCD → 0.0. Expected: All five sub-cases pass. SSM invalid returns zero. |  |  |  |

# 4  FADEC Test Cases

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-FADEC-001 | HLR-FADEC-001 | PASS | N1 Valid Decode — CFM56 |
| Prerequisites: CFM56_5B4P profile available Procedure: Encode Encode_BNR(84.6, 0.00391) as label 0x31, SDI=1, SSM=NORM. Call Decode_N1. Assert Status=OK, N1_Pct in 0..110, abs(N1_Pct-84.6) < 0.05. Expected: N1 decoded correctly within resolution tolerance. Status=OK. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-FADEC-002 | HLR-FADEC-001 | PASS | N1 Parity Error Rejection |
| Prerequisites: TC-FADEC-001 passed Procedure: Encode N1 word. Flip bit 1 (XOR 2). Call Decode_N1 on corrupt word. Assert Status=Parity_Error. Expected: Parity error correctly detected. N1 rejected. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-FADEC-003 | HLR-FADEC-002 | PASS | EGT TOGA Exceedance — CFM56 |
| Prerequisites: CFM56_5B4P profile available (EGT_Max_TOGA=950) Procedure: Call Check_EGT_Limit(EGT_C=980, Profile=CFM56_5B4P, Rating=TOGA, Eng_ID=1). Assert Kind=EGT_Exceedance_TOGA, Engine_ID=1. Expected: Alert correctly raised. EGT 980°C > 950°C TOGA limit. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-FADEC-004 | HLR-FADEC-002 | PASS | EGT Below Limit — No Alert |
| Prerequisites: CFM56_5B4P profile (EGT_Max_Cont=895) Procedure: Call Check_EGT_Limit(EGT_C=741, Profile=CFM56_5B4P, Rating=CRZ, Eng_ID=1). Assert Kind=No_Alert. Expected: No alert. 741°C is safely below 895°C CRZ limit. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-FADEC-005 | HLR-FADEC-002 | PASS | EGT Exceedance — A350 Trent XWB |
| Prerequisites: TRENT_XWB84 profile (EGT_Max_TOGA=1060) Procedure: Call Check_EGT_Limit(EGT_C=1080, Profile=TRENT_XWB84, Rating=TOGA, Eng_ID=1). Assert Kind=EGT_Exceedance_TOGA. Expected: Exceedance correctly raised using A350 engine limits. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-FADEC-006 | HLR-FADEC-005 | PASS | Freshness Thresholds — All Three States |
| Prerequisites: None Procedure: (1) Elapsed=200ms: Assert No_Alert. (2) Elapsed=600ms: Assert Parameter_Stale. (3) Elapsed=2500ms: Assert Parameter_Lost. (4) Clock wraparound (Current < Last): Assert Parameter_Stale. Expected: All four freshness states correctly identified. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-FADEC-007 | HLR-FADEC-003 | PASS | Thrust Rating All 7 Values |
| Prerequisites: None Procedure: For bit patterns 0b000 through 0b110: Assert decoded rating matches expected IDLE, REVERSE, CRZ, CLB, MCT, FLEX, TOGA. Test 0b111 (unknown): Assert IDLE returned (safe default). Expected: All 7 ratings decoded correctly. Unknown maps to IDLE. |  |  |  |

# 5  Autopilot Test Cases

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-AFCS-001 | HLR-AFCS-001 | PASS | Nominal AP Engagement |
| Prerequisites: A320 CFM56 profile available Procedure: Call Validate_AP_Engage(Pitch=2.0, Roll=5.0, IAS=280, Source=FMS, Already_On=False). Assert Engaged_OK. Expected: All conditions within envelope → Engaged_OK. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-AFCS-002 | HLR-AFCS-001 | PASS | Boundary — Pitch Exactly At Limit |
| Prerequisites: TC-AFCS-001 passed Procedure: (1) Pitch=20.0 (at limit): Assert Engaged_OK. (2) Pitch=20.001 (just over): Assert Rejected_Pitch_High. (3) Pitch=-10.0 (at limit): Assert Engaged_OK. (4) Pitch=-10.001: Assert Rejected_Pitch_Low. Expected: Boundary values correctly handled. Limit values accepted; over-limit rejected. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-AFCS-003 | HLR-AFCS-001 | PASS | Boundary — Roll Exactly At Limit |
| Prerequisites: TC-AFCS-001 passed Procedure: (1) Roll=30.0: Assert Engaged_OK. (2) Roll=30.001: Assert Rejected_Roll. (3) Roll=-30.0: Assert Engaged_OK. (4) Roll=-30.001: Assert Rejected_Roll. Expected: Absolute roll limit correctly enforced. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-AFCS-004 | HLR-AFCS-002 | PASS | Altitude Target Range — Multi-Aircraft |
| Prerequisites: All 6 aircraft profiles Procedure: For each profile: (1) Target_Alt=0: Assert OK. (2) Target_Alt=Max_Altitude_Ft: Assert OK. (3) Target_Alt=Max_Altitude_Ft+1: Assert Target_Alt_Too_High. (4) Target_Alt=-1: Assert Target_Alt_Negative. Expected: Altitude limits correctly enforced per aircraft type for all 6 profiles. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-AFCS-005 | HLR-AFCS-003 | PASS | AP Disconnect Alert — Unconditional |
| Prerequisites: None Procedure: Call Process_Disconnect(Was_Engaged=True). Assert Alert_Out.Kind /= No_Alert. Call 100 times in loop: Assert all 100 invocations raise alert. Call with Was_Engaged=False: Assert No_Alert. Expected: Alert raised 100/100 times when was engaged. No alert when not engaged. |  |  |  |

# 6  IRS Test Cases

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-IRS-001 | HLR-IRS-001 | PASS | Attitude Decode Range Clamping |
| Prerequisites: None Procedure: (1) Pitch=1.2°: Assert result in -90..90, abs(result-1.2)<0.01. (2) Roll=-15.5°: Assert result in -180..180. (3) Heading=350°: Assert result in 0..360. (4) Inject max data field: Assert result clamped to range. Expected: All attitude values within physical bounds after decode. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-IRS-002 | HLR-IRS-002 | PASS | Triple IRS Voting — Unanimous |
| Prerequisites: Three IRS units with valid data Procedure: Set IRS1 pitch=1.200, IRS2 pitch=1.210, IRS3 pitch=1.190 (all within 2° threshold). Call Vote_IRS. Assert result=Unanimous. Expected: Unanimous vote when all three agree within threshold. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-IRS-003 | HLR-IRS-002 | PASS | Triple IRS Voting — Single Miscompare |
| Prerequisites: TC-IRS-002 passed Procedure: Set IRS1=1.2, IRS2=1.21, IRS3=6.5 (deviate by 5.3° from others). Call Vote_IRS. Assert result=Single_Miscompare. Expected: IRS3 outlier detected. Result=Single_Miscompare. |  |  |  |

| Test Case | HLR Traced | Result | Title |
| --- | --- | --- | --- |
| TC-IRS-004 | HLR-IRS-002 | PASS | Primary Selection After Miscompare |
| Prerequisites: TC-IRS-003 passed Procedure: With IRS3 faulty (pitch=6.5), call Select_Primary. Assert result.Valid=True. Assert result.Pitch_Deg in -90..90 (not the faulty value). Expected: Primary selection returns valid data from agreeing units. |  |  |  |

# 7  Coverage Execution Procedure

- Run all tests with coverage: ./bin/aerosys_tests (built with --coverage)

- Generate coverage: gnatcov coverage --level=stmt+mcdc --annotate=xcov+ --output-dir=cov_report @aerosys_tests.sid

- Open cov_report/index.html and screenshot summary

- For any uncovered line: raise Problem Report (Category 2 unless justified)

- Record overall MC/DC percentage in SVTR Section 4