AEROSYS AVIONICS SYSTEMS LTD.

AeroSys 9000 Integrated Avionics Platform

TOOL QUALIFICATION PLAN

(TQP — DO-330)

| Document Number | AEROSYS-TQP-001 |
| --- | --- |
| Revision | A — Initial Baseline |
| Date | 2026-03-14 |
| DO-178C Level | Per DO-330 |
| Classification | PROPRIETARY — RESTRICTED |

# 1  Purpose

This Tool Qualification Plan (TQP) identifies the development tools used on the AeroSys 9000 project that require qualification under RTCA DO-330, Software Tool Qualification Considerations, and describes the qualification approach for each tool.

# 2  Tool Classification

Tools are classified by their potential to introduce or fail to detect errors in the deliverable software:

| TQL | Criteria | Examples on This Project |
| --- | --- | --- |
| TQL-1 | Output used directly as part of airborne software with no other verification | GNATprove (proofs replace testing), GNATcoverage (coverage evidence) |
| TQL-2 | Output used to eliminate/reduce a DO-178C activity | GNATstack (replaces manual stack analysis) |
| TQL-3 | Output used to automate a required DO-178C activity | GNATtest (test generation) |
| TQL-4 | Failure could prevent required error detection | Static analysers used as sole method |
| TQL-5 | Failure could introduce error but output is verified by other means | Compiler, linker, build tools — output verified by testing |

# 3  Tools Requiring Qualification

## 3.1  GNATprove — TQL-1

| Item | Value |
| --- | --- |
| Tool | GNATprove 24.0 (AdaCore) |
| TQL | TQL-1 |
| Rationale | SPARK proofs for DAL B components replace MC/DC test evidence per DO-333 Section 6.7. If GNATprove incorrectly reports a proof as discharged when it is not, an undetected defect could enter the deliverable. |
| Qualification Basis | AdaCore GNATprove Qualification Kit (SPARK 2014 TQL-1) — available from AdaCore under support contract |
| Qualification Activities | (1) Install per AdaCore qualification procedure; (2) Execute AdaCore-supplied qualification tests; (3) Review tool anomaly list against project usage; (4) Document qualification in TQR |
| Qualification Evidence | AEROSYS-TQR-GNATPROVE-001 (Tool Qualification Record — to be completed) |

## 3.2  GNATcoverage — TQL-1

| Item | Value |
| --- | --- |
| Tool | GNATcoverage 24.0 (AdaCore) |
| TQL | TQL-1 |
| Rationale | Coverage data produced by GNATcoverage is used as primary evidence of MC/DC structural coverage compliance. If GNATcoverage misreports coverage (claims covered when not), uncovered code could enter the deliverable without detection. |
| Qualification Basis | AdaCore GNATcoverage Qualification Kit (TQL-1) — covers xcov and xcov+ annotation modes used on this project |
| Coverage Modes Used | --level=stmt+mcdc (DAL B), --level=stmt+decision (DAL C), --level=stmt (DAL D) |
| Qualification Evidence | AEROSYS-TQR-GNATCOV-001 (to be completed) |

## 3.3  GNATstack — TQL-2

| Item | Value |
| --- | --- |
| Tool | GNATstack 24.0 (AdaCore) |
| TQL | TQL-2 |
| Rationale | GNATstack worst-case stack analysis is used to eliminate the need for manual stack depth measurement. If GNATstack underestimates the stack, a stack overflow could occur in the field. |
| Qualification Basis | AdaCore GNATstack Qualification Kit (TQL-2) |
| Usage | Run after each code baseline: gnatstack -P aerosys.gpr --output-format=gnatsymb |
| Independent Check | Stack depth results verified against manual call graph analysis for top 10 deepest paths |
| Qualification Evidence | AEROSYS-TQR-GNATSTACK-001 (to be completed) |

# 4  TQL-5 Tools — Operational Requirements

TQL-5 tools (compiler, linker, build tools, test frameworks) do not require formal qualification but must have their correct installation verified. For each TQL-5 tool:

| Tool | Version | Verification Method | Reference |
| --- | --- | --- | --- |
| GNAT Pro compiler | 24.0 | Execute known-answer test suite; verify compilation of aerosys.adb | AEROSYS-TQR-GNAT-001 |
| gprbuild | 24.0 | Verify deterministic output: two builds from same source produce byte-identical binary | AEROSYS-TQR-GPRBUILD-001 |
| GNATtest | 24.0 | Verify generated test skeletons match subprogram signatures | AEROSYS-TQR-GNATTEST-001 |
| GNATcheck | 24.0 | Verify rule violations are reported correctly using known-violation test files | AEROSYS-TQR-GNATCHECK-001 |
| AUnit | 24.0 | Run AUnit self-test suite; verify pass/fail results reported correctly | AEROSYS-TQR-AUNIT-001 |
| Git | 2.43 | Verify tag creation, retrieval, and signed-tag verification | AEROSYS-TQR-GIT-001 |

# 5  Tool Qualification Schedule

| Tool | TQL | Qualification Start | Qualification Complete | Owner |
| --- | --- | --- | --- | --- |
| GNATprove | TQL-1 | 2026-06-01 | 2026-09-01 | Cert Lead + AdaCore support |
| GNATcoverage | TQL-1 | 2026-06-01 | 2026-09-01 | Cert Lead + AdaCore support |
| GNATstack | TQL-2 | 2026-09-01 | 2026-12-01 | Lead Engineer |
| GNAT Pro + gprbuild | TQL-5 | 2026-06-01 | 2026-07-01 | Build Engineer |
| GNATtest + AUnit | TQL-5 | 2026-06-01 | 2026-07-01 | Test Lead |