AEROSYS AVIONICS SYSTEMS LTD.

Integrated Avionics Software Platform

PLAN FOR SOFTWARE ASPECTS OF CERTIFICATION

(PSAC)

| Document Number | AEROSYS-PSAC-001 |
| --- | --- |
| Revision | A — Initial Release |
| Date | 2026-03-14 |
| Classification | PROPRIETARY — RESTRICTED |
| DO-178C Level | Level B (primary) / Level C/D (displays) |
| Certification Authority | FAA AIR-6 / EASA NAA |
| Prepared By | AeroSys Software Certification Team |
| Approved By | DER / ACO Representative (TBD) |

This document is submitted to the FAA Aircraft Certification Office and/or EASA National Aviation Authority

for review and approval prior to commencement of software development activities.

# Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| A | 2026-03-14 | Cert Team | Initial PSAC release for DER/ACO review |
| — | TBD |  | Post-DER review updates |

# 1  Purpose and Scope

## 1.1  Purpose

This Plan for Software Aspects of Certification (PSAC) describes the means by which AeroSys Avionics Systems Ltd. will demonstrate compliance with RTCA DO-178C, Software Considerations in Airborne Systems and Equipment Certification, for the AeroSys 9000 Integrated Avionics Software Platform.

This document is submitted to the certification authority as required by DO-178C Section 11.1 and serves as the primary agreement document establishing the software life cycle, software levels, standards, and development environment to be used throughout the project.

## 1.2  System Description

AeroSys 9000 is an integrated avionics software platform providing the following functions:

- Flight envelope monitoring and parameter display (PFD, ND, ECAM/EICAS)

- ARINC 429 bus interface — receive and decode data from FADEC, ADC, IRS, FMS, AFCS, ILS/MMR, TCAS, and system buses

- REST API data distribution layer for ground station and maintenance access

- Multi-aircraft type profile management (A318/A319/A320/A321, B737 NG/MAX, A350, A380)

- Autopilot mode and target management interface

- Crew alerting system (CAS) — WARNING / CAUTION / ADVISORY

- Flight management interface — route display, performance data, FMS targeting

## 1.3  Applicability

This PSAC applies to all software identified in Section 4 of this document. It covers software hosted on the aircraft-side processing unit and the associated Vercel Edge serverless layer used for ground and maintenance access.

## 1.4  Related Documents

| Document | Number | Title |
| --- | --- | --- |
| RTCA | DO-178C | Software Considerations in Airborne Systems and Equipment Certification |
| RTCA | DO-278A | Software Integrity Assurance Considerations for CNS/ATM Systems |
| RTCA | DO-330 | Software Tool Qualification Considerations |
| RTCA | DO-332 | Object-Oriented Technology and Related Techniques |
| SAE | ARP-4754A | Development of Civil Aircraft and Systems |
| SAE | ARP-4761 | Guidelines and Methods for the Conduct of the Safety Assessment Process |
| RTCA | DO-160G | Environmental Conditions and Test Procedures for Airborne Equipment |
| ARINC | ARINC 429 Part 1 Rev 17 | Mark 33 Digital Information Transfer System |
| FAA | AC 20-115D | Airborne Software Development Assurance Using EUROCAE ED-12 / RTCA DO-178 |
| EASA | AMC 20-115D | Airborne Software Development Assurance Using EUROCAE ED-12C |
| AeroSys | AEROSYS-SRS-001 | Software Requirements Specification |
| AeroSys | AEROSYS-SDP-001 | Software Development Plan |
| AeroSys | AEROSYS-SVP-001 | Software Verification Plan |

# 2  Software Overview

## 2.1  Software Functions

The AeroSys 9000 software is organised into the following functional partitions. Each partition is independently allocated to a software level based on the safety assessment described in Section 3.

| Software Component | DAL | Ada Package | Function |
| --- | --- | --- | --- |
| FADEC Bus Interface | B | AeroSys.Bus (FADEC) | Receive and validate FADEC ARINC 429 words; forward N1, EGT, FF, oil data |
| TCAS RA Interface | B | AeroSys.Bus (TCAS) | Process TCAS resolution advisory commands from ARINC 429 bus |
| Autopilot Mode Manager | B | AeroSys.Server (AP) | Accept and validate autopilot engage/disengage, mode, and target commands |
| ARINC 429 Decode Layer | C | AeroSys.ARINC429 | Word decode, parity check, BNR/BCD/DIS format parsing for all buses |
| IRS Data Interface | C | AeroSys.Bus (IRS) | Receive and process attitude, position, acceleration from IRS buses |
| ADC Data Interface | C | AeroSys.Bus (ADC) | Receive and process speed, altitude, temperature from ADC buses |
| FMS Interface | C | AeroSys.Bus (FMS) | Receive route, performance, and targeting data from FMS buses |
| Crew Alerting System | C | AeroSys.Datastore | Manage WARNING/CAUTION/ADVISORY alert states and acknowledgement |
| PFD/ND Display Logic | C | AeroSys.Server | Format and serve telemetry data for cockpit display rendering |
| Aircraft Profile Mgr | D | AeroSys.Aircraft | Maintain engine limits, performance envelopes, bus configurations |
| REST API Layer | D | AeroSys.Server | Serve JSON responses to authorised external REST clients |
| Bus Monitor Display | E | api/_arinc429.js | Ground display of ARINC 429 bus words — no flight effect |
| Vercel Serverless Layer | E | api/*.js | Stateless serverless functions for ground/maintenance access |

## 2.2  Software Architecture

The software is implemented in Ada 2022 (aircraft-side) and JavaScript (ground layer). The architecture provides strict partitioning between safety levels through Ada's Protected Object mechanism, fixed-size buffers, and no dynamic memory allocation in DAL B/C components.

Key architectural properties:

- All inter-task communication through Ada Protected Objects (AeroSys.Datastore) with priority-ceiling protocol

- No dynamic memory allocation (no heap use) in DAL A/B/C components

- No unbounded recursion — all call graphs statically bounded

- Fixed-size string buffers throughout (String(1..N) constraints)

- Constrained numeric subtypes enforce physical ranges at compile time

- All ARINC 429 decoding validates parity before accepting data

## 2.3  Hardware Description

The target hardware platform is defined in the Hardware Qualification Plan (AEROSYS-HQP-001, TBD). The software is developed and verified on the following representative configuration:

- Processor: x86-64 or ARM Cortex-R52 (certification target TBD with customer)

- RTOS: VxWorks 653 or INTEGRITY-178 tuMP (DO-178C compliant)

- ARINC 429 interface: Astronics AceXtreme or DDC BU-65590 series cards

- Memory: 512 MB RAM minimum; no virtual memory in safety partitions

# 3  Certification Basis and Software Levels

## 3.1  Certification Authority

| Primary Authority | FAA — Aircraft Certification Office (ACO), Transport Aircraft Directorate |
| --- | --- |
| European Validation | EASA — per Bilateral Aviation Safety Agreement (BASA) |
| Applicable FAA AC | AC 20-115D (DO-178C compliance) |
| Applicable EASA AMC | AMC 20-115D |
| DER Engagement | FAA-designated DER to be assigned prior to Phase 2 |
| ACO Point of Contact | TBD — to be established at PSAC submission meeting |

## 3.2  Safety Assessment Summary

The software level assignments in Section 2.1 are derived from the System Safety Assessment (SSA) conducted per ARP-4754A and ARP-4761. The Functional Hazard Assessment (FHA) identified the following top-level failure conditions driving DAL assignments:

| Failure Condition | Effect | Severity | DAL | Software Function |
| --- | --- | --- | --- | --- |
| Incorrect thrust command transmitted to FADEC | Uncommanded thrust change — possible loss of control | Catastrophic | B* | FADEC Bus Interface |
| TCAS RA command not correctly processed | Failure to avoid collision | Catastrophic | B* | TCAS RA Interface |
| Incorrect autopilot targets accepted | Hazardous exceedance — severe workload increase | Hazardous | B | Autopilot Mode Manager |
| ARINC parity error not detected | Invalid data used for display decisions | Major | C | ARINC 429 Decode Layer |
| Incorrect IRS data displayed | Pilot orientation error — major workload | Major | C | IRS Data Interface |
| FMS route data corrupted | Incorrect navigation guidance displayed | Major | C | FMS Interface |
| Alert state incorrectly managed | Alert suppressed — delayed crew response | Major | C | Crew Alerting System |
| Aircraft profile limits incorrect | Erroneous display of exceedance margins | Minor | D | Aircraft Profile Mgr |
| REST API unavailable | Loss of ground access — no flight effect | No Safety Effect | E | REST API Layer |

* Note: FADEC and TCAS functions receive data only from the ARINC 429 bus and do not issue commands directly to flight-critical actuators in this architecture. The DAL B assignment reflects the potential for incorrect display driving incorrect pilot action. If future versions issue direct FADEC commands, these functions will be elevated to DAL A.

## 3.3  Alternatives and Deviations

No deviations from DO-178C are planned at this time. Any deviations identified during development will be documented as Issue Papers and submitted to the ACO/DER for approval prior to implementation.

# 4  Software Life Cycle

## 4.1  Life Cycle Overview

The AeroSys 9000 software life cycle follows a sequential waterfall model with feedback loops, as permitted by DO-178C Section 2. The life cycle consists of the following phases:

| # | Phase | Duration (est.) | Key Outputs |
| --- | --- | --- | --- |
| 1 | System Requirements & Safety | 6–12 mo | FHA, FMEA, FTA, SSA, system requirements, DAL assignments, PSAC approved |
| 2 | Software High-Level Requirements | 4–8 mo | SRS (HLRs), traceability to system requirements, HLR review records |
| 3 | Software Design (LLRs) | 4–8 mo | SDD (LLRs), architecture diagrams, interface specifications, data flow |
| 4 | Coding | 8–18 mo | Baselined Ada/JS source, coding standard compliance records, code reviews |
| 5 | Unit & Integration Testing | 12–24 mo | LLT procedures, LLT results, HIL test results, MC/DC coverage data |
| 6 | System Verification | 4–8 mo | HLT procedures, HLT results, integration test results, traceability complete |
| 7 | Configuration Management | Ongoing | Baselines, CCB records, problem reports, SCI |
| 8 | SAS & Certification Submission | 4–8 mo | SAS, full artifact package, DER review, ACO approval |

## 4.2  Transition Criteria

Each phase transition requires satisfaction of the following criteria before proceeding:

- All planned reviews for the phase are complete with no open Category 1 findings

- All required documents are baselined in the CM system

- Problem reports from the phase are dispositioned (closed or accepted with rationale)

- DER/ACO hold points (if any) have been cleared

- Independence requirements have been satisfied (reviewer ≠ original author)

## 4.3  Feedback Loops

The following feedback loops are explicitly planned and do not require a formal deviation:

- Requirements errors discovered during design: update SRS, re-review affected sections, re-trace to system requirements

- Design errors discovered during coding: update SDD, re-review affected sections

- Requirement gaps discovered during testing: raise PR, update SRS and SDD, re-test

- Coverage gaps discovered during analysis: add test cases, re-run coverage, update SVTR

# 5  Software Development Standards

## 5.1  Requirements Standards

Software requirements shall be written to satisfy the following quality attributes per DO-178C Section 5.1.1:

- Unambiguous — each requirement has exactly one valid interpretation

- Completeness — all system requirements allocated to software have corresponding HLRs

- Verifiable — each requirement can be tested by inspection, analysis, or test

- Consistent — no conflicting requirements

- Traceable — each HLR traces to one or more system requirements; each LLR to one or more HLRs

- Achievable — implementation feasibility confirmed by design

## 5.2  Design Standards

The AeroSys architecture standards are:

- Partitioning: DAL levels shall not share memory or task contexts without explicit analysis

- No global mutable state visible across package boundaries — all access through Protected Object (AeroSys.Datastore)

- Stack bounds: maximum stack depth for each task shall be determined by static analysis and verified by test

- Timing: worst-case execution time (WCET) for all DAL B/C interrupt handlers shall be bounded

## 5.3  Coding Standards

DAL A and B components: SPARK 2014 subset with GNATprove formal verification

DAL C components: Ada 2022 with the following restrictions:

- No dynamic memory allocation (no new/unchecked_deallocation)

- No recursive subprograms

- No uninitialized variables — all objects given explicit initial values

- All exception handlers shall be documented with rationale

- No use of System.Address arithmetic outside of hardware interface layer

- Maximum cyclomatic complexity per subprogram: 15

- Maximum subprogram length: 200 lines

DAL D and E components (JavaScript/Vercel layer): ESLint with airborne-js ruleset; no dynamic eval.

## 5.4  Verification Standards

Reviews shall use the checklists in SVP Appendix A. Each checklist item shall be marked Pass, Fail, or N/A with written justification for N/A.

Testing shall achieve the following structural coverage objectives:

| DAL | Statement | Decision/Branch | MC/DC | Notes |
| --- | --- | --- | --- | --- |
| A | Required | Required | Required | + structural coverage of data coupling |
| B | Required | Required | Required | GNATprove for SPARK components |
| C | Required | Required | Not required |  |
| D | Required | Not required | Not required |  |
| E | Not required | Not required | Not required |  |

# 6  Software Development Environment

## 6.1  Development Tools

| Tool | Version | TQL | Usage / Qualification Basis |
| --- | --- | --- | --- |
| GNAT Pro (AdaCore) | 24.0 | TQL-5 | Ada 2022 compiler — qualification data provided by AdaCore Qualification Kit |
| GNATprove | 24.0 | TQL-1 | SPARK 2014 formal verification — produces proof obligations for DAL B |
| GNATtest | 24.0 | TQL-5 | Unit test skeleton generation — output reviewed and supplemented manually |
| GNATcoverage | 24.0 | TQL-1 | Source coverage analysis — MC/DC and statement/branch |
| gprbuild | 24.0 | TQL-5 | Build system — deterministic, reproducible builds |
| GNATcheck | 24.0 | TQL-5 | Coding standard enforcement — DO-178C Ada subset rules |
| GNATstack | 24.0 | TQL-1 | Static stack analysis — worst-case stack depth |
| Node.js | 20 LTS | TQL-5 | JavaScript runtime for Vercel layer (DAL E only) |
| Git | 2.43 | TQL-5 | Configuration management — all baselines tagged |
| DOORS / Polarion | TBD | TQL-5 | Requirements management and traceability |
| Jenkins | TBD | TQL-5 | CI/CD pipeline — automated build and test execution |

## 6.2  Tool Qualification

Tools that could introduce undetected errors into the deliverable software (TQL-1 through TQL-4) require qualification per DO-330. GNATprove and GNATcoverage are TQL-1 tools. Their qualification is addressed in the Tool Qualification Plan (AEROSYS-TQP-001, TBD).

TQL-5 tools (compilers, build tools, test harnesses) must have their correct installation verified and their outputs reviewed. The GNAT Pro Qualification Kit provided by AdaCore covers TQL-5 qualification for GNAT Pro, gprbuild, GNATtest, GNATcheck, and GNATstack.

## 6.3  Configuration Management

All software life cycle data shall be managed under CM per the Software Configuration Management Plan (AEROSYS-SCMP-001). Key CM controls:

- All source files, requirements documents, design documents, test procedures, and test results shall be stored in Git with signed tags for each baseline

- Change Control Board (CCB) approval required for any change to a baselined artifact

- Problem reports shall be tracked in a dedicated PR system; no PR shall be closed without disposition

- Software Configuration Index (SCI) shall be maintained and submitted with the SAS

# 7  Schedule and Hold Points

## 7.1  Certification Milestones

| M | Milestone | Target Date | Deliverable / Criterion |
| --- | --- | --- | --- |
| M1 | PSAC submitted to ACO/DER | 2026-04-01 | This document; ACO/DER engagement established |
| M2 | PSAC approved by ACO/DER | 2026-06-01 | Written DER concurrence; development authorised |
| M3 | SRS baseline (HLRs complete) | 2026-10-01 | AEROSYS-SRS-001 Rev A baselined; HLR review complete |
| M4 | SDD baseline (LLRs complete) | 2027-03-01 | AEROSYS-SDD-001 Rev A baselined; design review complete |
| M5 | Code baseline | 2027-12-01 | All source baselined; code review complete; no Category 1 PRs open |
| M6 | LLT complete (unit tests) | 2028-06-01 | All LLTs passed; MC/DC coverage achieved for DAL B |
| M7 | HLT complete (system tests) | 2028-12-01 | All HLTs passed; traceability matrix 100% complete |
| M8 | SAS submitted | 2029-03-01 | AEROSYS-SAS-001; complete artifact package |
| M9 | Type Certificate / STC | 2029-09-01 | FAA/EASA approval — subject to authority schedule |

## 7.2  Hold Points

The following hold points require DER/ACO review and written approval before proceeding:

- HP-1: PSAC approval — no software development shall commence until ACO/DER approval of this document

- HP-2: SRS approval — no detailed design shall commence until HLRs are approved

- HP-3: Coverage analysis review — DER review of MC/DC evidence before HLT commences

- HP-4: SAS review — DER review of complete artifact package before certification authority submission

# 8  Additional Considerations

## 8.1  Previously Developed Software (PDS)

No previously developed software (PDS) is used in DAL A/B components. The AeroSys 9000 codebase is developed from scratch for this project.

The following pre-existing components are used in DAL C/D/E roles and require credit analysis:

- Ada runtime library (GNAT Pro RTS) — DAL C credit requires runtime qualification (AdaCore RTS qualification data available)

- Vercel serverless runtime (Node.js) — DAL E only; no credit required

## 8.2  Commercial Off-The-Shelf (COTS) Software

No COTS software with safety function is used. The ARINC 429 decoding library is developed in-house (AeroSys.ARINC429 package).

## 8.3  Multi-Version Dissimilar Software

Not applicable for the current architecture. If redundant dissimilar software channels are added in future, this section will be updated with the dissimilarity analysis per DO-178C Section 12.3.3.

## 8.4  User-Modifiable Software

The aircraft type profile data (AeroSys.Aircraft package) contains modifiable parameters. These parameters are controlled through the CM system and require CCB approval for any change. Field modifications are not permitted.

## 8.5  Option-Selectable Software

Software options (aircraft type selection) are controlled through protected configuration data managed per the CM plan. All option combinations are tested as part of the verification program.

## 8.6  Formal Methods

SPARK 2014 formal verification (GNATprove) will be applied to all DAL B components. GNATprove is used to prove:

- Absence of runtime errors (ATC — absence of runtime checks)

- Correct data flow (information flow analysis)

- Subprogram contracts (Pre/Post conditions on all public subprograms)

GNATprove qualification is addressed in the Tool Qualification Plan per DO-330.

# 9  Compliance Statement and Approval

The undersigned parties agree that this PSAC accurately describes the approach to be used for demonstrating DO-178C compliance for the AeroSys 9000 Integrated Avionics Software Platform and commit to executing the activities described herein.

| Prepared by — AeroSys Software Certification Team | DER Review and Concurrence |
| --- | --- |
| Name: ________________________________ Title: ________________________________ Signature: ____________________________ Date: _________________________________ | DER Name: _____________________________ DER Certificate No.: ___________________ Signature: ____________________________ Date: _________________________________ |
| ACO / NAA Point of Contact | Quality Assurance Witness |
| ACO Name: ____________________________ Office: ________________________________ Signature: ____________________________ Date: _________________________________ | QA Name: _____________________________ Organisation: __________________________ Signature: ____________________________ Date: _________________________________ |